# Abstract

Object detection in autonomous driving degrades sharply under foggy conditions, where the atmospheric scattering model couples scene radiance, transmission, and depth in a way that confounds conventional detectors. Existing approaches either dehaze the image as a preprocessing step and feed the restored output to a detector, or train a detector directly on foggy images without exploiting restoration cues. Both paradigms are suboptimal: the former propagates restoration errors into detection, while the latter ignores the physically grounded information that dehazing provides. We propose the Weather-Resilient Detection Unified Network (WRDNet), a multi-branch architecture that jointly performs restoration, detection, and depth estimation while adaptively fusing features across the restoration and detection streams. At the core of WRDNet is a Feature Selection Gate (FSG) that learns a per-pixel, multi-scale weighting between dehazed and original features, allowing the network to exploit restoration cues where they are beneficial without committing to a single restored image. We further introduce a Depth-Guided FSG (DG-FSG) that uses estimated monocular depth to actively modulate the fusion, exploiting the physical coupling between fog severity and scene depth. To address the synthetic-to-real domain gap, we propose a multi-level frequency-aware domain-adaptation strategy combining Fourier Domain Adaptation, DCT-based feature alignment, and a novel FSG-consistency loss. We also exploit the multi-density structure of the Foggy Cityscapes dataset to train across three scattering coefficients, tripling the effective data and enforcing fog-severity invariance. Extensive experiments on Foggy Cityscapes, ACDC, Foggy Driving, and Foggy Zurich demonstrate that WRDNet consistently outperforms sequential dehaze-then-detect baselines and prior joint approaches, while the learned gating maps provide physically interpretable evidence that the model adapts its fusion policy to scene depth and fog density.

Index Terms — object detection, image dehazing, domain adaptation, feature fusion, adverse weather, autonomous driving.

# I. Introduction

## Research Gaps and Contributions

A systematic review of the fog-removal and adverse-weather detection literature reveals several persistent limitations that motivate our work. We synthesize these gaps and state the corresponding contributions of WRDNet.

### Identified Research Gaps

G1 — The sequential dehaze-then-detect paradigm is suboptimal. Classical prior-based methods, beginning with the Dark Channel Prior (DCP) [1], and their lightweight variants [2, 3, 4], treat restoration as a standalone preprocessing step that produces a single dehazed image for a downstream detector. This decoupling is fundamentally limited: the atmospheric scattering model couples restoration and detection through the transmission map, yet the two stages are optimized independently. As a consequence, restoration errors propagate directly into detection. Over-aggressive dehazing can hallucinate artifacts in near-field regions (where fog is thin and the prior is unreliable), while conservative restoration leaves distant objects obscured. The detector is forced to operate on a single, fixed restored representation that cannot adapt to the spatially varying reliability of the dehazing output. Recent reviews of defogging for object detection [5, 6] confirm that this sequential paradigm consistently underperforms joint approaches, motivating feature-level integration.

G2 — Restoration quality is not aligned with detection objectives. Dehazing methods are predominantly optimized for image-quality metrics such as PSNR, SSIM, BRISQUE, and NIQE [2, 7, 8, 9]. These perceptual metrics do not correlate with downstream detection performance: an image that scores well on structural similarity may still be suboptimal for object localization, particularly for small and distant objects that are most vulnerable under fog. The Cycle-Defog2Refog framework [7] and the Gamma-CNN enhancement method [8] both optimize reconstruction fidelity without any task-aware signal, while the histogram-equalization family [9, 10] applies content-agnostic contrast enhancement. This metric misalignment means that even a “perfectly” dehazed image, by perceptual standards, does not guarantee improved detection, and can in fact degrade it by amplifying noise.

G3 — Single-density training limits fog-severity robustness. Foggy-driving benchmarks are typically constructed at a single scattering coefficient β [2, 4, 11], causing detectors to memorize a specific atmospheric appearance. In real driving, visibility varies continuously with fog density, and a detector trained at one operating point fails to generalize across the continuum. The multi-density nature of the Foggy Cityscapes dataset [12] — which renders each scene at β∈{0.005,0.01,0.02} — is rarely exploited, leaving a significant source of physically grounded training signal unused.

G4 — The synthetic-to-real domain gap is unaddressed. Methods trained exclusively on synthetic fog [2, 3, 7] degrade on real foggy images due to the distribution shift between simulated and physical fog. Prior work either ignores this gap entirely or relies on large, manually curated real datasets that are expensive to obtain. The Cycle-Defog2Refog framework [7] explicitly acknowledges that its physics-based refogging model fails under dense fog, and that its real dataset (MRFID) is too small (200 scenes) for robust generalization. Domain-adaptation methods for adverse weather [13, 14] address this gap but focus on a single alignment level, leaving multi-level alignment unexplored.

G5 — Depth information is underutilized. The atmospheric scattering model explicitly couples fog severity to scene depth (tx=e−βdx), yet existing joint dehazing-detection methods either ignore depth entirely or output it as a parallel prediction that never feeds back into the detection decision. Methods such as DEHRFormer and DCL estimate depth as a byproduct of dehazing but do not use it to modulate the fusion of restored and original features. This is a missed opportunity: depth is a physically meaningful prior that indicates where dehazing is most beneficial.

G6 — Numerical instability in joint training. Jointly optimizing restoration and detection is notoriously unstable. Unbounded feature magnitudes from the restoration branch can destabilize the detection head’s box regression, and mixed-precision training can introduce overflow-induced NaNs. Prior work does not systematically address these practical obstacles, which are critical for reproducible, stable training of multi-task adverse-weather detectors.

### Contributions of WRDNet

C1 — Joint, feature-level fusion via the Feature Selection Gate (FSG). Rather than committing to a single restored image, WRDNet keeps the detection backbone on the original foggy image and injects dehazed cues at the feature level through a learned, per-pixel gating mechanism. This directly addresses G1 and G2: the network learns where dehazing is beneficial rather than trusting it uniformly, and the gate is trained end-to-end with the detection objective rather than a perceptual metric.

C2 — Multi-density fog training. We exploit the fact that Foggy Cityscapes [12] provides each scene at three scattering coefficients β∈{0.005,0.01,0.02}, tripling the effective training data and enforcing fog-severity invariance. This addresses G3 by compelling the network to learn representations robust across the atmospheric continuum.

C3 — Multi-level frequency-aware domain adaptation. We bridge the synthetic-to-real gap (G4) through three complementary mechanisms: Fourier Domain Adaptation (FDA) at the input level, DCT-based feature alignment at the representation level, and a novel FSG-consistency loss at the output level. This multi-level strategy aligns the two domains without requiring real annotations.

C4 — Depth-guided feature fusion (DG-FSG). We introduce the first mechanism in which estimated monocular depth actively modulates how restored and original features are combined, exploiting the physical coupling between fog and depth (G5). The gate learns to trust dehazed features more for distant objects and original features more for nearby objects, producing a physically interpretable fusion policy.

C5 — A numerically stable joint-training recipe. We address G6 through a combination of bfloat16 mixed precision (which avoids the overflow-induced NaNs of float16), a magnitude clamp on the fused features, and a two-phase training schedule that first establishes a robust detection baseline before enabling the fusion and domain-adaptation modules.

# II. Related Work

The design of WRDNet is grounded in a rich body of prior work on image dehazing, adverse-weather detection, and domain adaptation. We organize this review around the principal research directions that motivate our approach, and we position WRDNet relative to each.

Classical dehazing. Early methods rely on hand-crafted priors, most notably the Dark Channel Prior (DCP) [1] and its lightweight variants [2, 3, 4]. These methods estimate the transmission map and atmospheric light from statistical priors and recover the scene radiance via the atmospheric scattering model. While computationally efficient, they are prone to failure under dense fog and overexposed regions, and they do not adapt to the downstream detection task.

Learning-based dehazing. Deep learning methods replace hand-crafted priors with learned mappings. Cycle-consistent adversarial networks [7] enable unpaired dehazing, while transformer-based methods such as DehazeFormer [15] and TCL-Net [17] achieve state-of-the-art restoration quality. However, these methods are optimized for perceptual image-quality metrics rather than downstream detection performance.

Adverse-weather detection. Recent work enhances detectors for foggy conditions through attention and multi-scale fusion, including YOLOv8s-WAMNet [16] and CAST-YOLO [14]. These methods improve robustness but do not jointly optimize restoration and detection, and they do not exploit depth information.

Domain adaptation. To bridge the synthetic-to-real gap, Fourier Domain Adaptation (FDA) [21] aligns input-level spectral statistics, while feature-level alignment methods such as AdaDCP align representations in the frequency domain. These methods address the domain gap but focus on a single alignment level.

Depth estimation. Monocular depth estimation methods such as DPT and MiDaS provide dense depth maps from a single image. Prior joint dehazing-detection methods such as DEHRFormer and DCL estimate depth as a byproduct of dehazing but do not use it to modulate feature fusion.

# III. Methodology

## Problem Formulation

We address the task of object detection under foggy driving conditions. Let I∈RH×W×3 denote a foggy RGB image captured by an onboard camera, and let Y={bi,ci} denote the set of ground-truth detections, where bi∈R4 is the bounding box and ci∈{0,…,C−1} is the class label over C semantic categories. Our objective is to learn a mapping fθ:I↦Y that remains accurate across a continuum of atmospheric conditions.

Fog degrades the observed radiance through the atmospheric scattering model [22]:

Ix=Jx tx+A 1−tx,

where Jx is the clean scene radiance, A is the global atmospheric light, and tx∈0,1 is the transmission map. Under a homogeneous atmosphere, the transmission decays exponentially with scene depth dx:

tx=e−β dx,

with β>0 the scattering coefficient (fog density). This formulation reveals a central tension for detection: aggressive dehazing can recover texture in the far field but may amplify noise and hallucinate artifacts near the camera, whereas conservative restoration preserves fidelity but leaves distant objects obscured. A detector that commits to a single restored representation therefore operates at a fixed point on this accuracy–fidelity trade-off.

## Weather Resilience of WRDNet

We define weather resilience as the capacity of a detector to sustain accurate performance across a continuum of atmospheric conditions, as opposed to a single operating point. WRDNet realizes this property through four complementary mechanisms that jointly address the spatial, severity, depth-conditional, and domain dimensions of weather variability.

- Spatial resilience through adaptive feature fusion. Fog exhibits pronounced spatial non-uniformity: near-field regions undergo only mild degradation, whereas distant regions are heavily obscured. A detector that commits to a single restored representation cannot accommodate this spatial variation. The Feature Selection Gate (FSG) learns a per-pixel weighting αx that interpolates between dehazed and original features at every location. In regions where dehazing is reliable (distant, fog-obscured areas), the gate favors restored features; where dehazing risks introducing artifacts (near-field, thin fog), it retains the original features. This spatial adaptivity constitutes the first pillar of weather resilience.

- Severity resilience through multi-density training. The scattering coefficient β varies continuously in real driving, spanning light mist to dense fog. A detector trained at a single β memorizes a specific atmospheric appearance and generalizes poorly. WRDNet exploits the multi-density structure of Foggy Cityscapes, which renders each scene at β∈{0.005,0.01,0.02}. This enforces fog-severity invariance, compelling the network to learn representations that remain discriminative across the entire visibility continuum rather than at a single operating point. This constitutes the second pillar of weather resilience.

- Depth-conditional resilience through DG-FSG. The atmospheric scattering model couples fog severity to scene depth via tx=e−βdx. WRDNet exploits this physical prior by conditioning its fusion policy on estimated monocular depth. The Depth-Guided FSG (DG-FSG) learns to favor dehazed features for distant objects and original features for nearby objects, yielding a fusion policy that is physically consistent with the scattering model. This depth-conditional behavior constitutes the third pillar of weather resilience.

- Domain resilience through multi-level domain adaptation. Synthetic fog differs from real fog in its spectral and textural statistics. WRDNet bridges this synthetic-to-real gap through a multi-level domain-adaptation strategy that aligns the two domains at the input level (FDA), the feature level (DCT alignment), and the output level (FSG consistency). This ensures that the learned representations transfer to real-world fog, constituting the fourth pillar of weather resilience.

Collectively, these four mechanisms enable WRDNet to preserve detection accuracy across varying fog density, spatial extent, and domain shift, which is the defining property of a weather-resilient detector.

## Architectural Overview

We propose the Weather-Resilient Detection Unified Network (WRDNet), a multi-branch architecture that jointly performs restoration, detection, and depth estimation while adaptively fusing features across the restoration and detection streams. The complete model is illustrated in Fig. 1 and comprises four principal components:

- Restoration branch — a lightweight DehazeFormer-T [15] transformer that estimates the clean image J and exposes multi-scale encoder features.

- Detection branch — a YOLOv11s [25] detector whose backbone operates on the original foggy image to preserve spatial resolution and small-object cues.

- Feature Selection Gate (FSG) — a learned, per-pixel gating mechanism that fuses restoration and detection features at three scales.

- Depth decoder — a lightweight progressive upsampling head that estimates monocular depth to guide the fusion (DG-FSG variant).

A key design decision is that the detection backbone consumes the original foggy image rather than the dehazed output. This preserves the native resolution and high-frequency content required for detecting small and distant objects, which are precisely the most vulnerable under fog. The restored representation is injected at the feature level through the FSG, allowing the network to selectively exploit dehazed cues where they are beneficial without committing to a single restored image.

## Feature Selection Gate (FSG)

The central contribution of WRDNet is the Feature Selection Gate, which learns a spatially varying interpolation between restoration and detection features. Let Fr∈RC×Hs×Ws and Fo∈RC×Hs×Ws denote the restoration and original feature maps at scale s, respectively. The FSG produces a gating map α∈0,11×Hs×Ws via a small convolutional gating network gϕ:

α=σgϕFr; Fo,

where ⋅;⋅ denotes channel-wise concatenation and σ is the sigmoid activation. The fused feature is then computed as a convex combination:

Ff=α⊙Fr+1−α⊙Fo,

where ⊙ is the Hadamard product. Because α∈0,1, the fused feature lies on the line segment between the two branches, guaranteeing that the gate can never amplify or invert the input statistics. Intuitively, α→1 selects the dehazed representation (beneficial for distant, fog-obscured regions), while α→0 retains the original features (beneficial for near-field regions where dehazing may introduce artifacts).

To enrich the gating context, we integrate a Cross-Dimensional Multi-Scale Attention (CDMSA) module that computes channel, spatial, and cross-scale attention over the concatenated features. The cross-scale term propagates information from coarser scales to refine the gating decision at finer scales.

Stability analysis. A critical practical concern is the numerical stability of the fused features when propagated through the detection head. The YOLO detection head employs Distribution Focal Loss (DFL) [23], which predicts a discrete distribution over box-edge offsets. If the fused features exhibit unbounded magnitude, the DFL logits can saturate, causing the predicted box edges to collapse to a degenerate configuration. To prevent this, we apply a magnitude clamp after normalization:

Ff=clampBNFf,−τ,τ,

with τ=10. This bounds the feature magnitude to a range compatible with the pretrained detection head while preserving the relative ordering of the gated fusion. We also initialize the gating network’s final bias to −2.0, which yields σ−2.0≈0.12, biasing the gate toward the original features early in training and preventing the detection loss from spiking on unstable restored features.

## Depth-Guided FSG (DG-FSG)

For the depth-aware variant, we augment the gating network with a monocular depth estimate. A lightweight depth decoder D progressively upsamples a bottleneck feature to produce a dense depth map d∈RH×W. The depth encoding is resized to each scale and concatenated with the restoration and original features:

α=σgϕFr; Fo; Ed,

where E is a depth encoder. This formulation exploits the physical prior that fog severity increases with depth: distant regions (large d) benefit more from dehazing, so the gate can learn a depth-conditional fusion policy. The depth decoder is supervised with a scale-invariant loss [24] on synthetic data.

## Multi-Density Fog Training

A central limitation of prior foggy-driving benchmarks is that they train on a single fog density β, causing the detector to memorize a specific atmospheric appearance. We instead exploit the fact that the Foggy Cityscapes dataset [12] provides the same scene rendered at multiple scattering coefficients β∈{0.005,0.01,0.02}. Training on all three densities simultaneously:

- Triples the effective training data, mitigating overfitting on the limited set of annotated scenes;

- Enforces fog-severity invariance, compelling the network to learn representations that are robust across the atmospheric continuum rather than a single operating point.

Formally, the training set is D=⋃β∈BDβ, where B={0.005,0.01,0.02} and Dβ is the set of images rendered at density β. This multi-density strategy is a principled, physically grounded form of data expansion that directly targets the task’s core challenge.

## Datasets

WRDNet is trained and evaluated on a combination of synthetic and real-world adverse-weather datasets. Table I summarizes the datasets, their roles, and their citations.

Table I: Datasets used in WRDNet.

| Dataset | Type | Split | Role | Citation |
| --- | --- | --- | --- | --- |
| Foggy Cityscapes | Synthetic fog (labeled) | train + val | Supervised detection + restoration + depth | [12] |
| ACDC | Real fog (unlabeled) | train | Domain adaptation (FDA, DCT, FSG-consistency) | [26] |
| ACDC | Real fog (labeled) | val | Validation (mAP monitoring) | [26] |
| Foggy Driving | Real fog (labeled) | test | Final evaluation (test mAP) | [12] |
| Foggy Zurich | Real fog (unlabeled) | test | Cross-domain generalization | [27] |

Synthetic training data. The primary supervised training signal comes from the Foggy Cityscapes dataset [12], which renders each Cityscapes scene at three scattering coefficients β∈{0.005,0.01,0.02}. We use both the train and val splits for training (3,475 base scenes × 3 densities = 10,425 images), since our final evaluation is on the disjoint ACDC and Foggy Driving datasets. Each synthetic sample provides a foggy image, its clear ground truth, a disparity-derived depth map, and instance-level bounding-box labels for eight driving-relevant classes.

Real-world domain-adaptation data. To bridge the synthetic-to-real gap, we use the ACDC dataset [26] as the unlabeled target domain during Phase 2. The ACDC train split provides real foggy images without labels, which drive the three domain-adaptation losses (FDA, DCT alignment, and FSG consistency). The ACDC val split, which has labels, is used to monitor mAP during training.

Real-world evaluation data. For the final evaluation, we use the Foggy Driving dataset [12], a standard benchmark of real foggy driving scenes with bounding-box annotations, and the Foggy Zurich dataset [27] to assess cross-domain generalization to an unseen real-world fog distribution.

## Domain Adaptation

To bridge the gap between synthetic fog and real-world fog, we incorporate unsupervised domain adaptation using real foggy images from the ACDC dataset [26]. The synthetic branch provides supervised detection supervision, while the real branch provides an unlabeled domain signal through three complementary losses:

- Frequency Domain Adaptation (FDA) [21] — swaps the low-frequency components of synthetic and real images to align their spectral statistics.

- Discrete Cosine Transform (DCT) alignment — aligns the feature distributions of the two domains in the frequency domain via a maximum-mean-discrepancy (MMD) loss.

- FSG consistency — encourages the gating maps produced on synthetic and real images to be consistent given their estimated fog density, promoting domain-invariant fusion.

The total training objective is a weighted sum of the detection loss Ldet, restoration loss Lrest, depth loss Ldepth, and domain losses:

L=Ldet+λrLrest+λdLdepth+λdomLdom+λfsgLfsg,

where the domain weight λdom is linearly ramped from 0 to its target value over the first epochs to avoid destabilizing the detector early in training.

## Two-Phase Training

### Rationale for Two-Phase Training

WRDNet is trained in two sequential phases, each with a distinct objective and data configuration. This staged strategy is motivated by three considerations.

First, the detection head is initialized from a randomly initialized 8-class Cityscapes head (replacing the pretrained 80-class COCO head), which must learn the driving-relevant class semantics from scratch. Training this head jointly with the restoration branch and the domain-adaptation modules from the outset would expose it to conflicting gradients from multiple objectives, destabilizing the box regression. A dedicated warmup phase isolates the detection objective, allowing the head to converge to a robust baseline before auxiliary tasks are introduced.

Second, the Feature Selection Gate (FSG) fuses restoration and detection features whose magnitudes differ substantially. When the restoration branch is frozen and the detection head is not yet calibrated, the fused features can exhibit unbounded magnitude that saturates the Distribution Focal Loss (DFL) [23] in the detection head, causing the predicted box edges to collapse to a degenerate configuration. Deferring the FSG to a later phase, after the detection baseline is established, mitigates this instability.

Third, unsupervised domain adaptation requires a well-initialized detector to provide a meaningful pseudo-supervision signal. Applying the domain-adaptation losses to an undertrained detector would align features that are not yet discriminative, yielding a poor adaptation. The two-phase schedule ensures that the detector is sufficiently trained before the domain-alignment objectives are activated.

### Phase 1: Detection Warmup

In the first phase, WRDNet is trained exclusively on the multi-density synthetic data from Foggy Cityscapes [12]. The restoration branch (DehazeFormer-T [15]) is frozen, and the Feature Selection Gate is bypassed, so the detection branch operates directly on the original foggy image features. This isolates the detection objective and establishes a robust baseline.

Objective. The training objective in Phase 1 is the detection loss only:

LPhase 1=Ldet,

where Ldet is the YOLO detection loss comprising box regression, classification, and Distribution Focal Loss components [23].

What the model learns. During Phase 1, the detection branch learns to localize and classify the eight driving-relevant classes (person, rider, car, truck, bus, train, motorcycle, bicycle) across the three fog densities β∈{0.005,0.01,0.02}. Because the training data spans multiple scattering coefficients, the detector learns fog-severity-invariant representations rather than memorizing a single atmospheric appearance. The multi-density data, combined with RandomScale and ColorJitter augmentation, mitigates overfitting on the limited annotated scenes.

Configuration. Table II summarizes the Phase 1 configuration.

Table II: Phase 1 (Detection Warmup) configuration.

| Hyperparameter | Value |
| --- | --- |
| Training data | Foggy Cityscapes (train + val, 3 densities) |
| Restoration branch | Frozen |
| Feature Selection Gate | Bypassed |
| Depth decoder | Disabled |
| Domain adaptation | Disabled |
| Epochs | 50 |
| Batch size | 24 |
| Learning rate | 1×10−4 |
| Optimizer | AdamW |
| Scheduler | Cosine annealing |
| Early stopping | Patience 10 on ACDC val mAP |

### Phase 2: Joint Fine-Tuning with Domain Adaptation

In the second phase, all components of WRDNet are unfrozen, the Feature Selection Gate is enabled (with the magnitude clamp), and unsupervised domain adaptation is applied using real foggy images from the ACDC dataset [26]. The synthetic branch provides supervised detection supervision, while the real branch provides an unlabeled domain signal.

Objective. The Phase 2 objective combines the detection, restoration, depth, and domain-adaptation losses:

LPhase 2=Ldet+λrLrest+λdLdepth+λdomLdom+λfsgLfsg,

where Lrest is the restoration loss, Ldepth is the depth loss, Ldom is the domain-alignment loss, and Lfsg is the FSG-consistency loss. The domain weight λdom is linearly ramped from 0 to its target value over the first epochs to avoid destabilizing the detector.

What the model learns. During Phase 2, the model learns to:

- Fuse restoration and detection features through the FSG, which learns a per-pixel weighting between dehazed and original features. The gate learns to favor dehazed features in distant, fog-obscured regions and original features in near-field regions.

- Align synthetic and real domains through the three domain-adaptation losses (FDA, DCT alignment, FSG consistency), enabling the learned representations to transfer to real-world fog.

- Estimate monocular depth through the depth decoder, which provides a depth-conditional signal to the DG-FSG variant.

Configuration. Table III summarizes the Phase 2 configuration.

Table III: Phase 2 (Joint Fine-Tuning with Domain Adaptation) configuration.

| Hyperparameter | Value |
| --- | --- |
| Training data | Foggy Cityscapes (synthetic) + ACDC (real) |
| Restoration branch | Unfrozen |
| Feature Selection Gate | Enabled (magnitude clamp τ=10) |
| Depth decoder | Enabled |
| Domain adaptation | FDA + DCT alignment + FSG consistency |
| Epochs | 120 |
| Batch size | 6 (paired synth + real) |
| Learning rate | 2×10−4 |
| Optimizer | AdamW |
| Scheduler | Cosine annealing |
| Early stopping | Patience 10 on ACDC val mAP |

### Optimization Details

Both phases use the AdamW optimizer with a cosine learning-rate schedule and gradient clipping at unit norm. Mixed-precision training is performed in bfloat16, which provides the exponent range of float32 (preventing overflow in the gating convolutions) while retaining the throughput benefits of reduced precision on modern tensor-core hardware. The learning rate in Phase 2 is set lower than in Phase 1 to avoid destabilizing the pretrained detection head when the domain-adaptation losses are introduced.

## References

[1] K. He, J. Sun, and X. Tang, “Single image haze removal using dark channel prior,” IEEE Transactions on Pattern Analysis and Machine Intelligence (TPAMI), vol. 33, no. 12, pp. 2341–2353, 2011.

[2] C.-C. Sun, N.-H.-H. Pham, A. A. Bryantono, and J.-W. Hsieh, “Lightweight computation single-image fog removal based on a new improved adaptive dark channel prior,” IEEE Transactions on Intelligent Transportation Systems (T-ITS), vol. 26, no. 11, 2025.

[3] “A fast method of fog and haze removal,” IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), 2016.

[4] X. Sang, Y. Yang, and X. Hou, “Fog removal method of slope monitoring image based on vision detection,” in Proceedings of the 37th Chinese Control Conference (CCC), 2018, pp. 1–6.

[5] I. Ogunrinde, “A review of the impacts of defogging on deep learning-based object detectors in self-driving cars,” in IEEE SoutheastCon, 2021.

[6] N. U. A. Tahir, Z. Zhang, M. Asim, J. Chen, and M. ELAffendi, “Object detection in autonomous vehicles under adverse weather: A review of traditional and deep learning approaches,” Algorithms, vol. 17, no. 3, 2024.

[7] W. Liu, X. Hou, J. Duan, and G. Qiu, “End-to-end single image fog removal using enhanced cycle consistent adversarial networks,” IEEE Transactions on Image Processing (TIP), vol. 29, pp. 7819–7833, 2020.

[8] H. Lim, J. Lee, H. Kim, H. Oh, and J. Paik, “Image enhancement for high-resolution visual contents,” in IEEE International Conference on Electronics, Information, and Communication (ICEIC), 2023.

[9] S. H. Gangolli and A. J. L. Fonseca, “Image enhancement using various histogram equalization techniques,” in IEEE Global Conference for Advancement in Technology (GCAT), 2019.

[10] Erwin, A. Nevriyanto, and D. Purnamasari, “Image enhancement using the image sharpening, contrast enhancement, and standard median filter (noise removal) with pixel-based and human visual system-based measurements,” in IEEE International Conference on Electrical Engineering and Computer Science (ICECOS), 2017.

[11] C.-Y. Lee, “Design a hardware applying fog removal algorithm using median dark channel prior for autonomous driving car,” in International Conference on Computational Science and Computational Intelligence (CSCI), 2023.

[12] C. Sakaridis, D. Dai, and L. Van Gool, “Semantic foggy scene understanding with synthetic data,” International Journal of Computer Vision (IJCV), vol. 126, no. 9, pp. 973–991, 2018. [Online]. Available: https://arxiv.org/abs/1708.07819

[13] S. Gharatappeh, S. Y. Sekeh, and V. Dhiman, “Weather-aware object detection transformer for domain adaptation (FogAwareAttention),” arXiv preprint arXiv:2504.10877, 2025.

[14] X. Liu, B. Zhang, and N. Liu, “CAST-YOLO: An improved YOLO based on a cross-attention strategy transformer for foggy weather adaptive detection,” Applied Sciences, 2024.

[15] Y. Song, Z. He, H. Qian, and X. Du, “Vision transformers for single image dehazing,” IEEE Transactions on Image Processing (TIP), vol. 32, pp. 1927–1941, 2023. [Online]. Available: https://arxiv.org/abs/2204.03883

[16] M. Jaiswal, K. K. Nagwanshi, M. Heenaye-Mamode Khan, U. Verma, and A. Taylor, “YOLOv8s-WAMNet: Enhancing robust vehicle detection under adverse weather via hybrid attention and multi-scale fusion in real time,” Scientific Reports, 2026.

[17] C. Tang and W. Lou, “TCL-Net: A lightweight and efficient dehazing network with frequency-domain fusion and multi-angle attention,” in ACCV, 2024.

[18] Y. Xie, H. Wei, Z. Liu, X. Wang, and X. Ji, “SynFog: A photo-realistic synthetic fog dataset based on end-to-end imaging simulation for advancing real-world defogging in autonomous driving,” arXiv preprint arXiv:2403.17094, 2024.

[19] A. Aryashad, P. Razmara, A. Mahjoub, S. Azizi, M. Salmani, and A. Firouzkouhi, “From filters to VLMs: Benchmarking defogging methods through object detection and segmentation performance,” arXiv preprint arXiv:2510.03906, 2026.

[20] N. Raza, M. A. Habib, M. Ahmad, Q. Abbas, M. B. Aldajani, and M. A. Latif, “Efficient and cost-effective vehicle detection in foggy weather for edge/fog-enabled traffic surveillance and collision avoidance systems,” Computers, Materials & Continua, 2024.

[21] Y. Yang and S. Soatto, “FDA: Fourier domain adaptation for semantic segmentation,” in Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition (CVPR), 2020. [Online]. Available: https://arxiv.org/abs/2004.05498

[22] H. Koschmieder, “Theorie der horizontalen Sichtweite,” Beiträge zur Physik der freien Atmosphäre, vol. 12, pp. 33–53, 1924.

[23] X. Li, W. Wang, L. Wu, S. Chen, X. Hu, J. Li, J. Tang, and J. Yang, “Generalized focal loss: Learning qualified and distributed bounding boxes for dense object detection,” in Advances in Neural Information Processing Systems (NeurIPS), 2020.

[24] D. Eigen, C. Puhrsch, and R. Fergus, “Depth map prediction from a single image using a multi-scale deep network,” in Advances in Neural Information Processing Systems (NeurIPS), 2014.

[25] G. Jocher and J. Qiu, “Ultralytics YOLOv11,” 2024. [Online]. Available: https://github.com/ultralytics/ultralytics

[26] C. Sakaridis, D. Dai, and L. Van Gool, “ACDC: The adverse conditions dataset with correspondences for semantic driving scene understanding,” in Proceedings of the IEEE/CVF International Conference on Computer Vision (ICCV), 2021.

[27] C. Sakaridis, D. Dai, S. Hecker, and L. Van Gool, “Model adaptation with synthetic and real data for semantic dense foggy scene understanding,” in European Conference on Computer Vision (ECCV), 2018, pp. 707–724.